API INFO

OVERVIEW

There are two main ports used in communicating with these modules. There is the  IR port, which can be changed by the user, and the discovery port, which is fixed.

The IR port defaults to 65432 and normally you would only expect to change this if you were using port forwarding with a number of modules. The discovery port is 30303.

The modules can store a limited number of IR codes internally. These when sent are always sent via the target.

It is not necessary for a code to be stored in the module. Code can be sent from a PC or HHD and acted upon immediately. All modules listen constantly on the IR port for an IR data string of the correct format, which they will then process. Just send in the correct format, by this means large IR code libraries can be built and used by a PC or network enabled device.

See further down this document for an explanation of the code string format and  "to send an ir code string"

The modules can receive an IR signal, convert to ASCII and send to two destinations in real time. The destination IP address1 (target) and destination IP address 2 (PC).

NEW for January 2011

Version 1.80 firmware for the Matrix.

This will now accept the X10 command strings sent as ascii to the module on the IR port.

The string can be up to about 160 chars in length and may contain multiple strings. It is also possible to store these strings in the memory locations.

Example send bu UDP on port 65432

$>28001B01 BONFF#$>28001B01 BDIM3C#$>28001B01 BDIM3C#$>28001B01 BDIM3C#

This is the string to turn on B1 and dim by 3 steps.

There is now also some support for macros. The macros can be enabled / disabled from the web page, also the start address can be set up. Once enabled if the start location is activated then the code in that location plus the code in the four following locations will be sent with a small delay between each. (about 30mS). These work when accessed via the API

NEW for September 2009

A number of changes and enhancements have been made to the API in order to accommodate the new IR Commander Matrix unit and to reflect customer requests.

With the Matrix storing up to 128 IR codes any API call that targeted the stored codes now has to send a hex 2 digit location - it was previously just a single digit.

A new command cmdM9 has been added, this is an extension of the existing cmdM commands and is simply used to set the MAC address (excluding colons) of the module as its host name. Keep in mind that restore to factory defaults would reset that.

Routing and repeats

Well the routing bit only relates to the Matrix but the format is the same for either so that the IR string can be used across the range.

It is now possible to tell the module to repeat the code a specified number of times with a specified gap between them.

Looking at a typical IR code string - this one is for a Sony

K 280D 096A 0269 025C 025C 025B 025E 025B 025D 04B4 025E 04B4 025D 025B 025E 025B 025D 04B5 025E 04B4 025E 04B3 025E 025B 025D 025B 2000

To add repeats to that string requires an additional string to be added at the end

K 280D 096A 0269 025C 025C 025B 025E 025B 025D 04B4 025E 04B4 025D 025B 025E 025B 025D 04B5 025E 04B4 025E 04B3 025E 025B 025D 025B 2000 00 4000 2

The “<space>00<space>4000<space>2” tell the module to send an additional 2 repeats of this string ie 3 in all and use a gap of 16384 uS between them. 4000 hex or about 16mS..

The first 00 is only applicable to the Matrix and is a bit screen with the lower four bits setting which output ports the IR will be routed through. A value of “00” is ignored and only the repeats acted upon.  So “11110011” will route the IR through ports 1 and 2 only.

Use windows calc to convert from binary to hex (scientific mode)  1110011 is F3 in hex

So “<space>F3<space>4000<space>2” would send our code out of ports 1 and 2 only.

New command cmdE??  Sent to a Matrix unit, causes the unit to emit an IR code. The code emitted can be learned and used as part of a macro for dynamic routing.  The 2 hex digits are a value between 00 and FF and are a bit screen. Only the lower four bits are used in the Matrix but both digits must be sent. Once learned the code can be part of a macro sent by a learning remote control. If the value of the hex digits was 01 then the only output port open would be port 1,  so the following IR code would be emitted only from that port.  The ports are automatically reset after the IR code. By including this code as part of a macro for each button press you can produce a remote control that will only operate a particular Set Top Box.

Please note this works as part of the KIRA system so will NOT operate for normal wired inputs

Also in the Matrix unit. The unit is capable of storing 128 IR codes. There are a couple of hidden web pages that can access these stored codes directly.

They can be accessed from any browser equipped device, so if you have WiFi on the local network, an iPhone or iPod touch, many WiFi enabled phones or pdas as well as wired devices like a PC etc.

To see the pages type http://<matrix IP address>/remote1.htm for a remote to access codes 1 to 32 and  http://<matrix IP address>/remote2.htm  for codes 33 to 64.

At the moment the pages are fairly basic but do provide the functionality.

Substitute bigremote1.htm for remote1.htm to see an iPhone friendly version or bigremote2.htm for remote2.htm.

Obviously if you were to set (HTTP) port forwarding up to point at the matrix it would be possible to access the pages from the web.

The remainder of this document is the ORIGINAL API.doc. Please read but note the changes mentioned above when using V1.70 or above firmware.

Finally a few changes have been made to the options web pages,  1 change forces either on – board or external IR receivers for the IRAnywhere modules.

There is now a check box for long duration code. This requires a bit of explanation. For most IR codes a typical burst or no – burst time is between 200 and 1500 uS and in this case a no-burst time of about 5000 uS is considered code over. There are a few IR codes that have up to 8000 or so uS of space or no- burst in the middle of the code.

Checking the long duration box changes the requirement used to determine that code is complete to about  12000 uS. This allows the firmware to work with the longer codes but as we have to wait at the end for 12000 us to determine that the code is complete, fast codes that would normally repeat quickly can confuse the system. Best to leave this unchecked unless you know you need it.

There is also a tidy code check box, this does some on the fly code averaging to even out the peaks and troughs of the code.. It does take about  10 mS so should only be used with code that repeats slowly – also not all codes like to be averaged in this way so just suck it and see.

COMMANDS

General commands

K (string) convert this string and and emit as IR - now!

cmdI set destination address 2 (PC) address my IP address (send to me)

cmdP set Port (caution! will loose connection)

cmdM set Mode. (Receiver, Target or StandAlone)

cmdT Transmit a stored IR code

cmdR Receive a code for storage.

cmdU Update stored memory

cmdB reBoot module

cmdD change Dhcp setting

cmdC Configure IP addreses and hostname

Discovery command list .

These all take place on the discovery port of UDP 30303

Commands received by modules

disD Standard Discovery command

disC Configure command

disI Request for IP address  of a target

disN Request a list of stored IR code Names.

disS Set target address (receiver only)

Responses sent

General discovery response.

disS Response to target address request, unicast.

disR Response to request for IR code list.

To send an IR code string to a module (emit ir)

To send an IR code string to a module you send a UDP packet to the IP address of the module with both the source and destination ports set to the IR port. If the string is in the correct format the module will process the string and output as IR. If successful it will return “ACK” to the senders IP address on the same port. Please note that routers do not guarantee delivery of UDP packets so on a busy network you may sometimes loose a packet  (Although this is rare in practice).

To receive an IR signal on a PC or HHD (learn ir)

The module can send a string representation of an IR signal to two destinations a “Target” typically another module for real time IR distribution, and a PC or HHD. A receiver module can send to both destinations while a stand-alone module or target can send to one (Target or StandAlone cannot send to another target). Modules are normally pre-set to Target, Receiver or StandAlone, but can be changed from the option web page or via an application

To avoid unnecessary net traffic modules don't attempt to send to the PC unless the option 'find PC' is checked on the options web page.

A developer can send a command to the module to force a change to the destination PC IP address, this also enables the find PC option.

For an application to learn IR you would need to have a module configured to send the ASCII text representation of the IR signal to your IP address, where your app can process and store it.

When learning IR, have the remote control a normal operating distance from the receiver module I.e. about 3 - 4 meters. They will give distorted timings if you have the remote too close.

Much of the necessary set-up can be done from the web page for the module, but it can also be done programmatically.

You need to have a module configured as either a receiver or stand-alone. If need be this can be done with the cmdM command.

Next you need to direct the second output nominally referred to as 'PC', to you. This is done with the cmdI command.

You should now be able to receive and learn IR codes. They will be received on the UDP IR port in the same format as you would send to a module. Once received you can send to a module to test.

COMMAND DETAILS

Commands are sent as ASCII text to the IP address of a module on the IR port, all commands will respond with an ACK if they are received OK.

cmdI  IP address - set the second destination (PC) address as my IP address (send to me)

Just send ASCII text 'cmdI' to the IP address of a module to set its destination PC IP address to the IP address of the sender.(your app)

Send with both source and destination ports set to the IR port  - default is UDP port 65432

cmdP P ort set

Send ASCII text 'cmdP12345' to the IP address of the module to set that module’s IR port to 12345. Note all subsequent communication will need to be on the new port!

Send with both source and destination ports set to the IR port  - default is UDP port 65432. Use the web page or discovery to find port if you loose it.

cmdM Mode set.

Send ASCII text 'cmdM0' to the IP address of a module to set it as a receiver. 'cmdM1' sets as a target and 'cmdM2' sets as stand-alone. You can also send cmdM3 to the module. This has the same effect as sending cmdM2 but will first copy the MAC address string, excluding the colons, to the hostname. This gives a very easy way of creating a unique host name for each module. The mode is then set as “2” stand-alone.

Send with both source and destination ports set to the IR port  - default is UDP port 65432

cmdT Transmit a stored IR code

Send ASCII text 'cmdT1' to the IP address of a module to send the code stored in location 1  ('cmdT2' to send code 2 etc.). If no code is stored in that location nothing will be sent.

Send with both source and destination ports set to the IR port  - default is UDP port 65432.

cmdR Receive a code for storage.

Send ASCII text 'cmdR1+code' to the IP address of a module to cause the attached code to be stored in that module and in location 1

Send with both source and destination ports set to the IR port  - default is UDP port 65432

Example using the code from stored code section further down this document

Send

cmdR1#Sny Cmdr Stp:K 280D 096A 0269 025C 025C 025B 025E 025B 025D 04B4 025E 04B4 025D 025B 025E 025B 025D 04B5 025E 04B4 025E 04B3 025E 025B 025D 025B 2000

Note the maximum length of code for storage is 49 bits. Longer code can be handled but not stored.

cmdD Dhcp on or off

Send ASCII text cmdD0 to turn off dhcp or cmdD1 to turn it on. Send to IP address of module on UDP IR port

cmdU  Update stored settings

Send ASCII text cmdU to the IP address of the module on the current UDP IR port.

To update all stored setting now.

The actual update will take place 2 ½ seconds from the command being sent, as it has to refresh the web page first.

Note:

Writing to the flash ROM in the microcontroller can only be done in large chunks, consequently all IP addresses and other setting are in one chunk and are all written at the same time. Any memory update will save all of these setting

cmdB reBoot

Send ASCII text cmdB to the IP address of the module on the UDP IR port to force a re-boot of the module. The unit will start re-boot 2 ½ seconds from the key press, as the module first has to serve the web page back to the user.

Note:

If using dhcp there is no guarantee that the module will be re-assigned the same IP address, but it usually will – allow about 30 seconds for the dhcp handshaking to take place before trying to access the module.

cmdCX

This is a general configure command for setting the IP addresses and hostname on the module

All have the same basic format cmdC +<identifying letter> + (String) <argument>  and are sent to the IP address of the module on the UDP IR port.

Example subnet mask. (note no spaces or termination char)

Send “cmdCS255.255.0.0” to set the subnet mask to 255.255.0.0

cmdCI = Modules own IP address (caution will  loose contact)

cmdCS = subnet mask

cmdCG = gateway

cmdC1 = DNS 1

cmdC2 = DNS 2

cmdCT = target

cmdCP = PC

cmdCH = hostname

cmdCA = MAC Address

Notes:

When updated these IP addresses are held in ram. You would need to send cmdU to save the addresses to ROM and make permanent

If DHCP is on, it is not possible to update the IP address for the module, the subnet mask, the gateway, and DNS server 1 and DNS server 2. This is because as soon as you change them they are re-set by the DHCP client in the module.

The IP address string is converted to IP address bytes by the micro in the module, invalid strings will simply be ignored although an ACK will still be sent if the command was actually received OK.

The hostname can be any ASCII chars equal to or above a space char 32 and below char 126. Any others are overwritten by a space. Note also that there are certain chars that are not legal in a hostname. for example the colon or backslash, forward slash etc.

The MAC address is the six bytes each as 2 digit ASCII separated by colons. E.g.  “cmdCA00:12:C2:90:AB:02” If the Mac address is changed the module will save settings and re-boot.

General notes on changing configuration

These modules will respond to http://hostname where hostname is the hostname of the module. But that gets stored in the PC's network cache and if the IP address or MAC addresses are changed, the cache will still contain the incorrect values and will need to be flushed. Typing 'nbtstat -R' into a command window should do the job. This can be a problem when using the web page to access the units after a re-boot if the web page is called using the hostname. This is also true if the DHCP server changes the module's IP address.

You may have noticed that the MAC address can be changed. This is unusual and is a 'feature' of the network controller we use. We would definitely not recommend changing the MAC address. But if you really must, it is normally only visible on your local network so as long as no two MAC addresses are the same it should not cause a problem. See IEEE Org for info on MAC addresses. If using dhcp changing the MAC will definitely cause the server to assign a different address.

Firmware upload

The firmware can be upgraded via the network. It is done via TFTP on UDP port 69 the usual one for TFTP.

It can be done from the command line in most operating systems along the lines of

TFTP <module IP address> put <file path>

Our java app also does a TFTP upload. And if you require the source for that just drop us an email. The java app will also try and detect the module's Mac address and current mode (receiver, target or stand alone) and modify the hex file so that the module is re-programmed to these values. If you use a command line TFTP program to upload the firmware you will need to ensure the mode and MAC addresses are set to the correct values as they are all programmed the same in the firmware

FIND MODULES

Discovery commands are sent as ASCII text on the discovery port 30303

disD   Discovery.

The modules have a discovery function built in.

To find which modules are on the network you would need to send a UDP packet to port 30303 with disD as the first 4 chars and with a broadcast IP address (255.255.255.255). This packet gets sent to any device on your local network.

Each device will respond with a sixteen-line answer, a CR LF follows each line

The received lines are:

Hostname

IP Address

MAC address

mode of unit

IR port

last IP address

Subnet mask

Gateway address

DNS 1 address

DNS 2 address

Target address

PC address

DHCP state

DDNS state

Firmware version

Text description of answer type

Example:

RECEIVER

192.168.100.3

00:50:C2:96:D0:00

0 = receiver

65432

192.168.1.32

255.255.255.0

192.168.100.77

194.74.65.68

194.74.65.69

192.168.100.1

192.168.100.2

DHCP = on

DDNS = off

1.60

Discovery reply

The modules also produce a discovery packet on re-boot or when the dhcp server assigns a new IP address

A developer can use this to get the IP addresses, MAC address, mode or port of any connected modules

Remember the user can change the host name! This defaults to Receiver, Target or standAlone but in a multi module system would typically be given meaningful names such as kitchen bedroom1 lounge etc. So that the user can access the module's web page via http://kitchen  etc.

disC  Configuration

Can be either sent as broadcast to change all modules or unicast to change just the one

Send disC as ASCII text to the module(s) on port 30303.

This moves the IP addresses on block to the subnet of the sender. This should only be used on a local network! It would typically be used when DHCP is not available or is disabled.

It changes the first 3 bytes of the IP address of gateway, 1st DNS, target address and the modules own address to match the first 3 bytes of the address from which the command was sent. All four bytes of the PC address are changed to match those of the sender.

Example

Old addresses

Gateway 					192.168.1.1

Module own IP				192.168.1.31

Destination PC				192.168.1.2

Target IP					192.168.1.32

Assume the modules are connected to a network 192.192.8.xxx based with a PC at 192.168.8.55, send “disC” from the PC on port 30303 to set first 3 bytes of IP addresses and four bytes of  PC IP address. Note all four bytes of the PC address are updated, so that the module can reply and send to you.

New addresses

Gateway 					192.192.8.1

Module own IP				192.192.8.31

Destination PC				192.192.8.55

Target IP					192.168.8.32

You will also get the normal discovery response followed by a re-boot discovery response as the module re-boots after saving the new addresses.

disI  IP address - search for IP address of a target

Broadcast on the usual port for discovery tasks, only a target or stand-alone module will respond to this. The module will respond with a disS.

Send disI as ASCII text to the broadcast IP address on port 30303.

disS  Send IP address Update Target address

Send disS192.168.1.31 as ASCII text to the IP address of the requesting module on port 30303.

Is sent by the module in response to the disI command, (although it could be sent by a PC) to the unicast address of the device that issued the request above, it has the format disI plus module’s own IP address as an ASCII string e.g. disI192.168.1.31. Strip off the 1st 4 chars to get the IP address as a string. This command is acted upon by a receiver module and has the effect of setting the receiver's target address to the IP address of the sender. To discover each other the modules make use of this command.

disN  Request a list of stored IR codes Names

Send disN as ASCII text to the module(s) on port 30303.

Sent as unicast to 1 module or broadcast to request a list of stored IR codes

disR Response to the above names request.

The First line is disR to identify as a response to request

Second line is IP address of sender in case request was broadcast

Third line is name of code stored in location 1

Forth line is name of code stored in location 2

Fifth line is name of code stored in location 3

Sixth line is name of code stored in location 4

IR CODE FORMAT

This code format is a simple ASCII text representation of the IR code. The first character in the string is a capital "K" followed by a space. The next four characters are the frequency expressed as a 2-digit hex number and number of bursts of carrier including the lead in burst followed by a space.

The remaining blocks of four ASCII characters are the times in uS in hex of the burst then no burst. All are separated by a space.

The final value is always 2000 and is the lead out no-burst time.

An example here is 12 bit Sony code

K 280D 097A 0253 0272 0241 04C9 0240 0273 0241 0272 0240 04CA 0241 0273 0241 0271 0242 04C9 0241 0272 0240 0273 0240 0273 0240 0273 2000

K followed by a space  = identifier

28 = decimal 40 = carrier frequency for Sony

0D = 13 = 12 bit data plus 1 for the lead in burst pair

The rest are times in uS

097A = decimal 2426 = 2426uS lead in burst of carrier
0253 = decimal 595 = 595 lead in no carrier or space-time.

Etc. till end of string, ignoring the K 280D there should be 13 (0x0D) x 2 blocks of four digits

UPLOAD CODE

To format a code for upload to the module for storage, add '#' name-string and a ':' (colon) to the code string.

The name can be no more than 14 chars long.

#Sny CmCdr Stp:K 280D 096A 0269 025C 025C 025B 025E 025B 025D 04B4 025E 04B4 025D 025B 025E 025B 025D 04B5 025E 04B4 025E 04B3 025E 025B 025D 025B 2000

When accepted by the module the leading # is stripped off and the text between the # and the colon is set as the name. Codes are uploaded using the web page or via an application with the cmdR command.

Any comments about this document or the IRAnywhere product range please email mark@keene.co.uk.

Please check our web site regularly for updates and responses to any comments etc.